TABLE 5-A

STREET DESIGN SPECIFICATIONS

RESIDENTIAL STREETS

Max. No. of	Permitted	Required	Min. Pave-	Max.	Min.

Dwelling Units/	On-Street	R.O.W.	ment Width1	Gradient	Gradient

Classification	Net Acre	Parking	(ft.)	(ft.)	(%)	(%)

Marginal Access	To 1.99	None	40	20	6	1.3

Local	2.0-4.50	Both Sides	50	30	6	1.0

Local Collector	4.50/Greater	Both Sides	50	34	6	1.0

Arterial2	Over 250

dwelling units

served	None	70	28	6	1.0

COMMERCIAL AND INDUSTRIAL STREETS

Permitted		Required	Max.	Min.

On-Street	Required	Pavement	Gradient	Gradient

Classification	Parking	R.O.W. (ft.)	Width (ft.)	(%)	(%)

Local	None	60	26	10	1.0

One Side	60	34	10	1.0

Both Sides	60	42	10	1.0

Collector	None	80	44	8	1.0

1	If curbs are required, as measured from back to back of curbs.

2	Parking lane width, add ten (10) feet.